# Architectural Control Committee Guidelines for Construction

Osage National Community Association

P. O. Box 2583

Lake Ozark, MO 65049

(573) 920-0200

onca.lakeozark@gmail.com

Revised June 14, 2025

OP 1738835.2

TABLE OF CONTENTS

I	Preface	5

- Building Information	5-13

- Review Process	l3-14

- Permit Processes	14

- Inspection Procedures	14-15

- Common Areas	15

- Trees	15-16

- Consolidating or Subdividing Lots	16-17

- Utilities	17-20

- Amendment Process	20

- Property	20

Exhibit A Sample Site Plan	21

TOPICAL INDEX

Amendment Process	20

Appeals (to reviews by ACC)	14

Approved Wastewater Treatment Systems	17

Architectural Control Committee	5

Back Flow Preventer	19

Building Information	5-13

Building Permit	13

Burn Permit	14

Central Sewer	17

Fees	9

Common Areas	15

Consolidating or Subdividing Lots	16-17

Corrective Action	15

Damage/Performance Deposit	9

Decks and Deck Railings	12

Design and Specifications (Lot and Home)	6

Driveways and Culverts	12

Electricity	20

Elevations	8

Exhibit A	21

Fences	12

Final Inspections	15

Floor Plans	6

Footing Inspections	15

Fuel Storage Tanks	19

Inspection Procedures	14-15

Landscape	11

Landscape Deposit	11

Membership	5

Permit Process	13-14

Poles	13

Property	20

Review Process	13-14

Right of Entry	14

Setbacks	8

Selection of Builder	5

Signs	13

Site Drawings	21

Site Plan	6-7

Site Preparation	9

Square Footage	8

Surveys	8

Swimming Pools	12-13

Tree Removal	15-16

Utilities	17-20

Variances	11

Water Service	19

# P R E F A C E

##### Purpose:

The Architectural Control Committee was established to review all improvements to any lot within the Osage National residential community. Diligent monitoring of improvements within the community assures the homeowner that the integrity of the development will not be compromised. The amended Declaration of Restrictions and Homes Association Declaration (the “Declaration of Restrictions”), which governs the land use, has been adopted to ensure the quality of living within this planned residential community, as well as maintain the peaceful sylvan characteristics of the area. The unique features of the community neighboring the Lake of the Ozarks require careful preservation of the woodland and wetland areas so as not to disturb the delicate balance of nature. The Architectural Control Committee is committed to this effort. Support from the homeowners and their contractors are necessary to allow Osage National to remain one of the premiere residential resort communities in the lake area.

##### Membership:

The Osage National Community Association's (“ONCA”) Board of Directors has appointed an Architectural Control Committee. The Committee meets by appointment on an as-needed basis in the Osage National Clubhouse, 400 Osage Hills Road, Lake Ozark, MO 65049 or as otherwise specified when an appointment is made.

# BUILDING INFORMATION

##### Overview:

ONCA’s Architectural Control Committee (ACC) has put together this booklet for information and review for anyone considering submitting plans for any and all exterior improvements or changes, including houses, garages, patios, pools, landscaping, painting, wastewater system improvements, etc.

Generally, plans will be reviewed by the ACC within 15 days of submission of a complete application. When submitting plans, all fees and applications must accompany them to be considered for review.

##### Selection of a Builder:

Choosing the right builder for your residence is a major decision. You should be personally comfortable with your contractor and feel he or she is competent. Speak to the references the contractor has provided as well as others for whom your contractor has built homes. Look closely at the workmanship of the homes your contractor has built. Ask the homeowners if the contractor has been available after the home was completed should minor repairs be needed. Do not hesitate in seeking references and comments other than those provided.

The Lake Ozark Builders Association can provide a list of contractors who are members of the Association. Check the local lending institutions for credit references. Request the firm or individual contractor to provide proof of workman's compensation, personal liability, and property insurance. Most owners' property insurance policies will not cover injuries to workers on your property during the course of construction or during any improvements.

Please be aware the property owner and contractors are responsible for following all city and state regulations regarding the construction of the home. Additionally, please call Dig Rite (800) DIG RITE before beginning construction.

The ACC or the Board of Directors does not provide a "preferred builders" list.

Any disputes that may arise between the owner and the contractor must be resolved privately. The ACC will NOT be involved as an arbitrator or for support of either party unless the dispute involves a direct violation to the approved plans or the rules and regulations for building within the development.

##### Design and Specifications:

    - General Information:

Manufactured homes are not authorized and will not be approved.

Every plan submitted should include a minimum of one (1) certified survey, two complete sets of floor plans and specifications, and two (2) copies of site plans. One (1) set of approved plans will be returned to the applicant, notations as to revisions and/or requirements, or exceptions for approval will be noted on the returned plans. One (1) set of plans will be retained for the ACC files. A City of Lake Ozark building permit will be required. The City will require a copy of the site plan and building plans.

#### Once the ACC approves plans, no modifications or alteration can be made without written ACC approval.

The Applicant is responsible for the validity and accuracy of the plans submitted for review by the ACC. The ACC is not responsible for errors or omissions in the plans and specifications reviewed or approved. There will be no variances granted at the building site during any phase of construction. The approved plans become a valid binding contract between the applicant and ONCA, therefore, it is necessary that all proposed changes be approved by the ACC in writing.

The plans should reflect all proposed improvements using a noted scale. Construction details must be provided for review. Type of siding, roofing materials, color scheme, building materials, etc., must all be provided at the time plans are submitted. Color swatches of base and trim color must be submitted with plans. The ACC may request an actual sample of any of the proposed building materials for the improvements. Should a sample be requested, the samples will become a part of the ACC file and may not be returned.

Concrete is to be conventionally poured with a raked, stamped or chopped finish. Other types of concrete designs must be submitted for ACC review.

##### Floor Plans:

Two (2) copies of floor plans must be submitted, on a minimum paper size of 24 inches x 36 inches, to the ACC. One copy will be retained for the ACC files and one copy will be returned to the owner, or the owner's designated representative. Electronic PDF format copies of plans are suggested as well to speed up the approval process and allow easy access for reference by the ACC. Floor plans must be accurate, must reflect the dimensions of the site plan, be to scale (with the scale noted), and depict the position of all water using appliances and appurtenances, as a part of the floor plan package, drawings showing front, rear, and side views of the structure are to be included. Said drawings shall accurately depict scaled dimensions, slopes, exterior materials, positioning of windows and shutters, chimneys, and facades. The floor plans must include square footage (entry level, lower level, upper level, and total). Square footage requirements are exclusive of decks, carports, garages, gazebos, porches, etc.

##### Site Plan:

Two (2) copies of the site plan, on a minimum size of paper of 24 inches x 36 inches, are to be submitted for review. All improvements must be incorporated into the site plan including, but not limited to:

- Dimensions of the lot, as referenced by accompanying certified survey;

- Topography with contour elevations drawn accurately in two-foot increments;

- Building envelope noted, setbacks defined;

- Structure, including garages, decks, patios, and courtyards depicted by a heavy, consistent line;

- Roof line or overhang depicted by a broken line;

- Location of grinder pump. Grinder pump models are approved by City of Lake Ozark;

- Location of mailbox on site;

- Tree locations and species of all trees over six (6) inches in diameter;

- Driveway location, dimensions, culvert location and size (minimum 15” pipe recommended but based on topography), existing finish grade elevations, type of material;

- Exterior heating and air conditioning units and location;

- Trash enclosures (must be maintenance-free materials);

- Floor elevations for each floor;

- Propane tank size and location. Propane tanks must be buried;

- Swimming pools, size and location, cross-section and elevations, and pump house (if included); swimming pools must meet safety requirements (i.e. BOCA Codes or Fire Protection District) with fenced area, if applicable; specifications of fence must be depicted;

- Proposed landscaping, retaining walls or terrace walls, plantings, boulders, etc.

- Site plans, which, in the opinion of ACC, do not provide sufficient detail, will not be approved;

- Location of any flagpoles;

- Playgrounds, pet confinement areas, and fences, where applicable;

- Any retaining walls for support of driveway, elevations, and cross-section;

- Surface drainage paths, means of diverting, or channeling;

- Landscape plans must be submitted for all spec homes;

#### See Exhibit A for sample site plan drawings.

- All exterior modifications must be submitted for ACC review.

##### Square Footage:

Minimum square footage requirements for residences shall be 1,600 square feet with at least 1,200 square feet on the main floor. In calculating the foregoing minimum square footage requirement, basements, porches, uncovered decks, patios and garages shall not be considered.

##### Setbacks:

All construction must be contained within the building envelope. Eaves and rooflines may extend a maximum of eighteen (18) inches into the setback. The setbacks are determined using the following lateral distances from the surveyed property lines:

Front:	25 feet from any lot line coincident with road right-of-way. Rear:	30 feet from rear lot line

Side:	10 feet from lot line

In cases where the city zoning requirements are more restrictive, they must be obeyed. The property owner should be aware of the existence of the ten (10) foot utility easements along the property lines of each lot when plotting the construction and landscaping.

##### Surveys:

A certified property survey and topography from a professional land surveyor, licensed in Missouri, must be submitted for construction. The survey must include the recorded subdivision, existing roads, utility easements, property lines with dimensions and bearings, lot number, subdivision and classification, orientation, date, owner of record, and any other features identifying the legal limits of the lot. Should any encroachments be included, said encroachments are to be identified and dimensioned as such on the survey.

After the survey has been completed, the property pins are to remain intact. It is unlawful to remove any property pins set by a licensed surveyor to delineate a property boundary. Pin location is to be marked to facilitate the necessary inspections. In order to clearly define the areas of activity, the property limits must be string-lined.

##### Elevations:

Determining grade elevations for proposed home construction is mandatory and must be signed and sealed by your surveyor. The contour elevations must be drawn accurately and to a noted scale, in no more than two (2)-foot increments

Profile drawings of the structure must incorporate the contour elevations of the lot. These elevations must be numerically noted as well as drawn on the elevations of the structure. Existing elevations are to be noted by means of a broken line; finish grade elevations noted by a heavy, consistent line. The topography of the property, as it relates to the structure, must be accurately detailed.

Elevations are to be provided if any structure or improvement is to be above grade, such as decks, landings, terrace walls, retaining walls, etc. Elevation of the floor of the lowest living area and the elevation of the original and finished grade at each corner of the structure must be included.

##### Site Preparation:

Limited clearing may be approved for site preparation in accordance with tree removal guidelines. Alteration of the existing grade is not allowed unless approved by the ACC. All property lot lines are to be string-lined after the licensed surveyor has established the property pins. Failure to do so can result in an inspection by the ACC not being approved.

The preservation of trees should be emphasized when designing and laying out the home. It is requested that all flowering trees be allowed to remain wherever possible. To protect trees during construction, it is suggested a "drip line" be created which consists of a fence or other identifying barrier erected around the tree. This "line" should follow the outermost branches of the tree as much as possible.

See Article VII. of this document, for further information on trees.

##### Fees:

    - Permit Fee:

A permit fee must be paid at the time plans are submitted. The permit fees are necessary to defray the administrative costs for operation of the ACC. Submittal fees are not refundable. Make a separate check payable to the Osage National Community Association and include “ACC Permit Fee” in the memo of the check.

The ACC permit fee shall be $500.00 per permit or other such fees as may be established from time to time. (Does not include any City of Lake Ozark permit fee(s)).

##### Damage Deposit:

A $2,500.00 damage/performance deposit must be posted for all new homes within the residential community. Make a separate check payable to the Osage National Community Association and include “Damage Deposit” in the memo of the check. Should there be damage from construction, the ACC will determine repair costs, and the refund will be adjusted accordingly. A copy of the estimate to correct the situation will be made available to the applicant upon request. All damage/performance deposits shall be returned to the titleholder of the property, regardless of who may have initially posted the damage deposit.

The damage/performance deposit will be refundable providing that all construction is completed in accordance with the terms and conditions of these guidelines and instructions. The ACC will issue a Stop Work Order for violations of the following articles:

- Uncovered dump trucks and other vehicles transporting sand, gravel, topsoil, fill, or any other loose material.

- Repeated violaitons of speed limits within Osage National.

- Lots approved for construction must be maintained in a professional, workmanlike manner, with all debris and trash confined to a trash receptacle of suitable size to accommodate the amount of rubbish for the lot. Contractors will ensure that all construction debris is removed or placed in an appropriate container by the end of day on Friday. Trash and debris left on a lot for a period of seven (7) days thereafter will be considered a violation. All trash and debris shall be disposed of off property and outside the limits of Osage National.

- Littering: Any driver, operator, or worker will be prosecuted to the fullest extent of the law for littering within the communities of Osage National. This may include a $1,000.00 fine and up to one year in jail imposed by the City, in addition to other measures deemed necessary by the Board of Directors.

- All fires for the disposal of trash and debris shall be attended to at all times, with appropriate means of extinguishing immediately available. In Osage National all controlled burns must be approved by the Lake Ozark Fire Protection District's Fire Marshall as well as the ACC

- No fill, rock, topsoil, sand, gravel, or other materials shall be placed on the road, road right- of-way, common area, or adjoining property at any time without express permission from all affected parties.

- All construction equipment should be parked on the lot and not impede traffic flow nor create a hazard for driving. Temporarily blocking one lane of traffic (i.e., delivering concrete, unloading and loading), shall be adequately identified for oncoming traffic. Construction vehicles left on the road or road right of way after hours or overnight will be marked with orange cones front and rear.

- Washing out cement trucks in the road rights-of-way, ditches, or adjoining property will not be allowed and may result in the forfeiture of all or part of the damage deposit. The cost for removing the wash will be borne by the contractor or property owner, as deemed appropriate by the ACC.

- Temporary individual sewage and personal sanitary facilities will be on all lots during all phases of construction.

- Any damage to adjoining property, roads, ditch lines, or common areas must be restored in a manner satisfactory to the ACC as well as the adjoining property owner (if applicable). If the restoration is not resolved in a timely manner, the damage deposit will be forfeited and utilized to reclaim the area. An additional deposit will be required to bring the total back to $2,500.00. This must be supplied by the contractor/property owner before resuming construction.

- Changes during the course of construction must be submitted to the ACC prior to commencing said changes.

- Neither advertisements nor promotional materials shall be displayed at the construction site’ Only the building permit issued by the ACC, City of Lake Ozark and the local fire protection district is to be displayed at the lot. A sign listing the builder and telephone number is allowed as a means of locating the builder in an emergency.

- A minimum twenty-four-hour advance notice is mandatory to schedule all inspections by the ACC representatives. This does not infer the inspection will be completed within the twenty-four-hour notice time. All property limits are to be identified by means of a string line to facilitate inspections. All property identification lines are to be left intact during the entire

course of construction. It is the contractor's or property owner's responsibility to inquire as to the status of the inspection. No concrete may be poured until verification has been given to the contractor or property owner that the inspection has been completed.

- All utility trenches must be backfilled and cables buried.

- Property pins must remain intact.

- The street in front of the construction lot and 100ft to either side will be cleaned of all loose debris, i.e., sand, gravel, dirt by the end of day Friday.

##### Landscape Deposit:

A Landscape Deposit of $2,000.00 is required with the submission of all plans. Make a separate check payable to the Osage National Community Association and include “Landscape Deposit” in the memo of the check. The Landscape Deposit will be escrowed with the ACC and will be released after the completion of the landscaping.

##### Banking:

All deposits may be placed in an interest-bearing account, with all interest from the funds in the account being applied to the operations account of the ACC.

##### Variances:

A reasonable variance may be petitioned before the ONCA Board of Directors from the documented provisions of the Restrictive Covenants and these Guidelines. A variance may not be approved for a self-inflicted violation caused by the contractor or property owner, including unapproved construction of a structure or improvement in violation of the building guidelines. The Board of Directors is not bound to approve or grant variances after the violation has occurred. Submitting an "as built" plan does not assure the "as built" variances will be approved. Approval from adjoining property owner(s) to allow violation of setbacks does not constitute approval by the Board of Directors, nor does it supersede the Board of Directors decision. Any home granted a variance by the Board of Directors must also obtain a variance from the City of Lake Ozark when applicable.

##### Landscape:

Site plans presented to the ACC shall include a landscape plan. Landscape plans must depict lawn space, existing trees and those that will be planted, and any above ground structures or grade elevations which will be added. Features to be shown include, but are not limited to, retaining walls, fountains, gazebos, flag poles, light standards, sidewalks, and flumes. Where topography allows, sod and irrigation are required for the front of a house and in the back where the property adjoins/faces the golf course. Variances from this need to be applied for and approved before deviating. Property owners are reminded that landscape maintenance is required by the Declaration of Restrictions. Please refer to Section X., A 2, of this document, including backflow preventer requirements for irrigation systems.

##### Driveways & Culverts:

Plans must be submitted depicting driveway location with dimensions; culvert location and size (as of this writing the City of Lake Ozark requires a fifteen (15) inch minimum), along with existing and finish grade elevations of driveway. Culverts are mandatory if there is an existing drainage ditch. Culverts must be of corrugated metal or plastic. It is the responsibility of the owner of record to maintain the culvert servicing the driveway to the residence, even if the culvert is within the road right-of-way.

No driveway apron shall be wider than the width of the garage. Hard surface driveways are required for all homes. Hard surfaces include concrete, asphalt and pavers.

Generally, no portion of any hard surface driveway may be constructed within the side yard setback. For driveway construction into the side lot setback to be approved, the driveway must be on existing grade. Should the driveway encompass a portion of the setback, appropriate and efficient methods of diverting the runoff must be included. Should a berm be utilized to support the driveway, the berm must be confined entirely within the boundaries of the lot. The driveway may not be supported by a retaining wall or similar structure where it encroaches into the side lot line setback.

Curbing must not exceed six inches (6 in.) in height above grade level.

The hard surface of the driveway must abut the road pavement and be flush with the existing pavement of the road at the point of juncture. Hard surfacing in anticipation for future road resurfacing will not be allowed.

Any constructed berm or landscaping within the side yard or backyard easement is the responsibility of the property owner. Should the utility easements be utilized by any utility company, the portion of the constructed berm or landscaping within the setback may need to be removed and repaired or replaced at the cost of the property owner.

##### Fences:

Plans must be submitted depicting location, dimensions, and type of fence proposed. Privacy fences are not allowed.

##### Decks and Deck Railings:

The ACC may allow a deck to extend beyond the building envelope. However, to minimize the extent of the encroachment into setbacks, all other possible means of locating the deck within the building envelope and maintaining a minimum deck size must have been exhausted prior to requesting the ACC to allow an encroachment. Variances will only be granted if a true hardship exists, based on the lot's configuration, topography and dimensions. Enclosed decks extending beyond the building envelope is strictly prohibited, however enclosing space under decks may be considered.

All walking platforms, including decks and martini decks, as defined by the City of Lake Ozark safety building codes, above grade elevation and as stipulated in these building codes, must be protected by a railing meeting appropriate codes. Pickets, if run vertically, must meet the appropriate city code. Guardrail protection must be afforded to all screened porches and similar structures above existing finish grade level as designated by the city building codes.

Decks are preferred to be maintenance-free materials (concrete, composite decking, etc.) as opposed to wood which deteriorates over time. If wood is selected as a deck material it should be kept in an aesthetically pleasing condition by sealing, staining, or painting on a regular basis.

##### Swimming Pools:

All pools must be submitted on a site plan, with a cross-section noting elevations attached. The pool and its appurtenances may not be located within 10 feet of the side or rear lot lines. All decking, retaining walls, railings, etc., must be submitted to the ACC in accurate detail. All swimming pools must also meet applicable codes for safety purposes. Please refer to Article X. A 2, of this document, for information on backflow preventer requirements. A swimming pool discharge permit will be required. Pool discharge may not be drained into the sanitary sewer system.

##### Poles:

All poles must adhere to the restrictions in Section 3 (4) of the Covenants and Restrictions. No flag poles, light standards, basketball goals, or other permanent structures or appurtenances over six feet in height are to be erected without prior ACC approval.

##### Builder Signs:

A sign listing the builder and telephone number is allowed as a means of locating the builder in an emergency. No advertisements nor promotional material shall be displayed at the construction site, as stipulated in Section 3(z) of the Declaration of Restrictions.

- The total dimensions of the sign may not be greater than 24 inches x 36 inches.

- The sign will contain, within the 24 inch x 36 inch area, the building permits affixed             there-to. The sign will be placed on its own post (and in no case on a tree), visible from the road and will be removed when construction is completed.

- The sign will contain only the company name, emergency phone number, and building permits. It will be of professional quality. No casually painted or hand-printed signs.

- Only one sign is allowed for each job site.

- Signs which do not meet the above criteria, or which are not, in the opinion of the ACC, the quality expected within the development, will be removed.

N. Modifications To Existing Structures

No approval from the ACC is required if changes to existing structures are of the same color, material, etc. For example, replacing shingles with the same color shingle would not require an ACC permit or review.  However, changes in color, material, landscaping, structural additions, pools, firepits, or other significant changes in design or appearance require ACC review and approval.

# REVIEW AND PERMITTING PROCESS

##### ARCHITECTURAL CONTROL COMMITTEE:

    - Review and Permitting:

Notice of the ACC's decision regarding approval or disapproval of plans will be sent to the applicant and owner of record within five working days of review. Should plans be disapproved for any reason, a meeting may be called to review the required revisions. A majority vote by the ACC members is necessary for approval.

Approval by the ACC will be in the form of a returned copy of the approved ACC Checklist, which will serve as the building permit from the ACC. No work is to begin until this is completed. Note that this does not constitute approval by the City of Lake Ozark, which may require additional permitting.

One set of approved plans will be returned at the time the permit is issued. The other set of plans is retained by the ACC to confirm that all construction is completed as approved. Building permits are issued for a ten (10) month period or for the amount of time the applicant feels is necessary for completion of the project, subject to Architectural Control Committee approval. Should construction not be completed by the date of expiration of the building permit, an extension must be requested and approved by the Committee. Any improvement not completed or upon which construction has ceased for ninety consecutive days or partially destroyed and not rebuilt within the life of the building permit, shall be considered a nuisance. The cost of repairs or completion shall be deducted from the deposit or if the cost exceeds the deposit amount, the balance shall be assessed to the property.

##### Appeals:

An adverse decision made by the ACC may be appealed to the ONCA Board of Directors within thirty days after receiving notice of such decision. Appeals made later than 30 days after receiving notice of an adverse decision may not be considered at the discretion of the Board of Directors.

.

##### OTHER REVIEWS:

City of Lake Ozark:

The City of Lake Ozark may require a permit for the project, and it is the responsibility of the contractor or property owner to determine whether or not a permit is necessary. The property owner or the owner's agent must go to the City Hall and display a copy of the stamped and signed building plans and the ACC permit. The City Hall telephone number is (573) 365-5378.

# OTHER NECESSARY PERMITS

## Burn Permits:

Open burning may be allowed with permission from the appropriate fire protection district, Lake Ozark Fire Protection District (573) 365-6407. Any unattended or unpermitted fire will be documented and the appropriate action instituted by the Fire Protection District. The Osage National Community Association may take corrective action against the property owner to include a stop work order, fine or both.

# INSPECTION PROCEDURES

## Overview:

During the construction process, several inspections must be completed by the ACC inspectors. At least a twenty-four-hour notice is required to schedule all inspections. Failure to allow sufficient time for scheduling the inspections may result in a delay of construction time. If possible, inspections will be completed so such delays do not occur. In the event the foundation of the structure is within five feet of any lot line setback, it may be necessary that a certified survey of the concrete or foundation is provided. It is the responsibility of the applicant to inquire as to the results of the inspection by the ACC. No work shall continue until said inspection has been approved. If disapproved, construction must cease until the inspection has been approved.

## Right of Entry:

Agents of ONCA have the right to enter any lot or tract at any reasonable time for the purpose of inspection, maintenance and repair of any part of the sewage treatment system, or for the purpose of inspecting for possible violation of the provisions of the Osage National Declaration of Restrictions or other rules and guidelines contained in this manual. The right of entry is granted to ONCA and its authorized representatives, its successors and representatives.

## Footing Location Inspections:

A footing location inspection is done to see that the construction is being performed as approved by ACC. No structural review is inferred, and general location is the only criterion used in ACC footing inspections. No concrete may be poured at any time without prior approval. All excavation is to be completed and forms correctly placed prior to requesting said inspection. The ACC may request a survey by a licensed professional land surveyor prior to pouring any concrete. A copy of the survey must be submitted for approval to be granted. No framing may begin until a copy of the certified foundation survey is received and approved by the ACC. An additional footing inspection by the City of Lake Ozark may be required.

When the footing inspection for the house is done, the owner's property will be reclassified from unimproved to improved for purposes of ONCA assessments.

## Final Inspection:

A final inspection will be completed at the time of expiration of the building permit, with results made available to the applicant. A copy of the occupancy permit from the City of Lake Ozark must be submitted prior to the ACC's final inspection. Should an additional inspection be necessary after the initial final inspection is completed, a charge of one hundred dollars ($100.00) for each additional inspection will be deducted from the posted damage/performance deposit to defray the costs of administration. Should the improvement be completed prior to expiration of the building permit, the applicant must notify the ACC office to schedule the final inspection.

## Corrective Action:

Failure to comply with the above rules and regulations may result in all or a portion of the damage deposit being forfeited. Any other course of action deemed necessary by the ONCA Board of Directors may be implemented to correct the violation. In the event a Stop Work Order is posted, all construction activity shall cease until the corrective action is completed and inspected by the ACC. Reinstatement of the building permit will require that all corrective action has been taken.

# COMMON AREAS

Areas designated as common, community, or access on the plat recorded in Miller County, Missouri are to remain in a natural state and are maintained by ONCA. These areas have been set aside for drainage, future ONCA improvements or amenities, sanctuary for wildlife and to maintain the woodland characteristics of the community. It is not permissible to make any improvement to these areas. Clearing the underbrush, grading, graveling, or any other alteration is prohibited unless approval is received by the Board of Directors or the ACC. Removal of any trees is also prohibited. Trees that have fallen or rotted may be removed by the property owner only if there is a potential danger to existing structures and prior approval by the ACC has been obtained. There are no permanent structures to be erected on any common area, including but not limited to, walkways, paths, retaining walls, etc. without prior written consent of the ACC.

# TREES

  - The ACC and its agents will review tree removal requests in a manner that takes the entire development into consideration, not just the individual lot. Tree removal is discouraged before actual development on a lot. Removal shall be limited to those trees that are diseased or in danger of falling and damaging property. If thinning of trees before development is desired a $75.00 permit fee must be paid and a $500.00 tree damage deposit posted. Additionally, the property owner must submit a plan for the disposal of the thinned trees, limbs and other materials. All trees over 6” in diameter (measured 2’ from the ground) that the property owner requests to remove shall be pre-marked with white paint; removal of pre-marked trees may be made only with the approval of the ACC.

- If trees are removed without a permit, the ACC may require the property owner to replace any trees removed immediately with a nursery grade replacement.

- Trees to be removed should be limited to those trees located in footprints of construction, diseased trees, dead trees, and deformed trees.

- The property owner will assume all responsibility for the removal of trees from his or her property. The property owner will pay all fees and fines and will be responsible for proper cleanup of any trees on their lot which are cut or which fall over.

- All trees, brush, limbs, or other debris consequent to clearing or removal shall be disposed of in an acceptable manner. Acceptable methods are burning (with permit), chipping and mulching, or carrying off site to a location where permission to dispose has been obtained. Failure to dispose of the cut material will be treated as a violation of a permit and can result in forfeiture of damage deposit and additional fines.

# CONSOLIDATING OR SUBDIVIDING LOTS

Lots may be subdivided if all portions of the lots subdivided are conveyed to the owners of an adjacent lot(s), thereby enlarging such lot(s) with the approval of ONCA. The boundary between the enlarged lot(s) shall constitute the new back and side lines for the purposes of meeting the building guidelines.

The subdivision of lots authorized by this section shall be effective only if the owners of the lot being subdivided and the owners of the lots being enlarged join in the execution of an indenture, setting forth the new boundaries of the enlarged lots. The indenture shall be approved in writing by the ONCA and will be recorded in the Office of the Recorder of Deeds, Miller County, Missouri.

If an Owner owns contiguous Lots, they may be combined into a single homesite but only upon obtaining the prior written approval of the applicable governmental authority and ONCA (each such approved combination of Lots being called a "Combined Lot"); provided, however, (i) for purpose of assessments, a Combined Lot shall be deemed only one Lot; (ii) all assessments with respect to a Combined Lot shall constitute a lien upon the entire Combined Lot; (iii) the Owner of each Combined Lot shall be entitled to the rights of only one Osage National Community Association Membership in respect of all such Lots so combined and (iv) the Owner shall relinquish any right to subdivide the Combined Lot in the future. After combining Lots, the Combined Lot shall remain as such, and the Owner(s) thereof shall not be permitted at any time to rent, sell, or otherwise transfer or convey less than all of such Combined Lot.

Lot combinations are intended to encourage building larger homes within the community that have difficulty being built within the setbacks of a single lot. Below are the requirements and restrictions regarding the combination of lots.

- A Dedication of Homesite form must be signed, notorized, and submitted to the ACC

- The Dedication of Homesite will not be considered until building plans are approved by the ACC

- Approval of lot combination will be based on whether the usage of the lots constitutes a single homesite or just a home on one lot with a vacant lot next door

- The lots must touch one another and will only be allowed to have one home on them

- A maximum of 2 lots may be combined

- All dues on the lots to be combined must be current at the time of dedication

- The owner will pay full dues on 2 lots until construction is completed, at which time the combined lots will be charged one set of dues

- Lot combinations granted by ONCA do not affect County/City taxes (including NID)

# UTILITIES

The following is provided for informational purposes only. Please check with the City or utility company for details.

## Wastewater:

##### Central Sewer Systems:

      - All residential lots are required to hook up to the central sewer system. A written application to the City of Lake Ozark for service (along with the connection fee) will be required prior to service being available. The service line from the structure to the property line is the responsibility of the customer:

      - The City of Lake Ozark will provide the connecting accessories. The applicant is advised to contact the City of Lake Ozark a minimum of forty-eight hours in advance during normal business hours of when the service is anticipated and installation thereof. The City of Lake Ozark must approve all connections prior to the trench being backfilled by the Applicant.

      - Whenever possible, the sewer service shall be brought to the building at an elevation below the basement floor. No building sewer shall be laid parallel to or within three feet of any bearing wall. The depth shall be sufficient to afford protection from frost and be laid at a uniform grade and in straight alignment insofar as possible. Changes in direction shall be made only with properly curved pipes and fittings.

      - The City of Lake Ozark will not be required to supply sewer service until each connection has been inspected and approved.

##### Sanitary Grinder Pump Sewage System:

It is the homeowner’s responsibility to ensure that the City’s requirements, including all amendments are adhered to with regard to the sanitary sewer system.

The homeowner may be required to purchase a grinder pump system to pump sewage from the home into the central sewage system.

#### Inspection

The City of Lake Ozarks Public Works Superintendent is permitted to enter private properties at any reasonable time for the purpose of inspection and observation to ensure that the grinder pump system is constructed in accordance with the City’s regulations.

#### Construction Permit

Before commencement of construction of a grinder pump system the owner shall first obtain a written construction permit signed by the City of Lake Ozarks Public Works Superintendent. The application for such permit shall be made on a form furnished by the City, which the applicant shall supplement by any plans, specifications, and other information relevant to wastewater discharges. A permit and inspection fee shall be paid to the City at the time the application is filed.

#### Operating Permit

Before commencement of operation of a grinder pump system, the owner shall first obtain a written permit signed by the City of Lake Ozarks Public Works Superintendent. The operating permit shall not become effective until the installation is completed to the satisfaction of the Superintendent. The Superintendent shall be allowed to inspect the work at any stage of construction, and in any event, the applicant for the operating permit shall notify the superintendent when the work is ready for final inspection, and before any underground portions are covered. The inspection shall be made within twenty-four (24) normal business hours, after receipt of notice by the Superintendent.

#### Connecting Costs

The costs and expenses incidental to the building sewer and grinder pump system installation and connection to the City’s wastewater facilities shall be borne by the property owner.

#### Separate Connections Required

A separate and independent building sewer and grinder pump system shall be provided for every home and building.

#### Building Sewer Design

The size, slope, alignment, construction materials, trench excavation and backfill methods, pipe placement, joining and testing methods used in the construction and installation of a building sewer shall conform to the building and plumbing code or other applicable requirements of the City of Lake Ozark.

#### Surface Runoff and Groundwater Drains

No person shall connect roof, foundation, areaway, parking lot, roadway, floor drains, swimming pools, hot tubs, or other surface runoff or groundwater drains to any sanitary sewer.

Roof, foundation, areaway, parking lot, roadway, or other surface or groundwater drains shall discharge to natural outlets or storm sewers.37

#### Conformance to Applicable Codes

The connection of a building sewer into a sanitary sewer shall conform to the requirements of the building and plumbing code or other applicable requirements of the City of Lake Ozark.

The connection of a surface runoff or groundwater drain to a storm sewer or natural outlet designed to transport surface runoff or groundwater drainage shall conform to the requirements of the applicable building codes or other applicable requirements of the City.

#### Connection Inspection

The applicant for a sanitary sewer connection permit shall notify the City of Lake Ozarks Public Works Superintendent when such sewer is ready for inspection prior to its connection to the City’s facilities. Such connection and testing as deemed necessary by the Superintendent shall be made under the supervision of the Superintendent.

#### Excavation Guards and Property Restoration

Excavations for sanitary sewer installation shall be adequately guarded with barricades and lights so as to protect the public from hazard. Streets, sidewalks, parkways, and other public property disturbed in the course of the work shall be restored in a manner satisfactory to the City of Lake Ozark.

#### Basis for Charges

The City of Lake Ozark shall collect sewer service charges for the use of the services rendered by the sanitary sewage system.

## Water Service:

##### Backflow Preventers:

The City of Lake Ozark requires that property owners install backflow preventers in water lines, including but not limited to water lines, swimming pools, and irrigation. Please contact the City of Lake Ozark at 573-751-4594 or at 365-5378.

##### Metered Water:

Metered water service is available to all lots at Osage National. Property owners should apply to the City of Lake Ozark.

Installation of water service shall be coordinated with electrical utility installation and located in the same trench. It is the customer's responsibility to provide the water service line and conduit for Ameren UE’s power cable to the structure. After the meter is set, a three-quarter inch male iron pipe thread will be needed to connect to the meter. It is the contractor's or applicant's responsibility to backfill after the connection.

Any requests for turning water off at the meter to repair a leak must be made to the City of Lake Ozark. It is strictly forbidden for a customer, or a plumber employed by the customer, to turn water service on or off at the meter. Personnel are available to turn service on and off during normal business hours. The cost to repair any damage done to the meter by the customer or the customer's representative may be billed to the customer. The City of Lake Ozark retains ownership of the water meter. It is the responsibility of the customer to make certain that the meter does not freeze in the winter months.

In some areas of the water distribution system, water pressure is considerably higher than necessary. The installation of a pressure reducing valve is suggested in these areas.

## Fuel Storage Tanks:

##### Above Ground Storage:

No above ground fuel storage is allowed.

##### Underground Propane Storage:

Underground propane storage will be allowed with the approval of ACC. The designated utility corridors or setback easements shall not be utilized for underground tank burial. The tank and all appurtenances shall be in accordance with the Lake Ozark regulations and be approved by the district's Fire Marshall. The location of underground propane storage tanks shall be noted on the site plan. Any deviations from the dimensions must obtain Fire Marshall’s approval prior to submission to the ACC.

A copy of said installation inspection and yearly inspections shall be forwarded to the ACC for inclusion in the ACC's files.

## Electricity:

Power is supplied to all of Osage National by Ameren Missouri (573) 365-9203. Standard procedure is to coordinate installation of power in the same trench as the water line.

# AMENDMENT PROCESS

The provisions of Osage National Community Association’s Architectural Control Committee Guidelines shall be binding upon all properties listed in the Declaration of Restrictions. This guideline for home builders can be amended at any time by ONCA.

# PROPERTY

The property covered by these guidelines is described in Exhibit A of Declaration of Restrictions. Additional property may be made subject to the guidelines upon designation by the Developer, its successor or assigns.

### EXHIBIT A SAMPLE SITE PLAN

PLAT OF A SURVEY

LOT 141 OAKMONT VILLAGE CITY OF LAKE OZARK MILLER COUNTY, MO

r	·r-·-r	r.. ...r· ..._

l

I	I	l.	l	\  --i;--,--.-·1·-y '

/,	)l1:1J,'\\

I

I	I	I	I	I  J	, .. \

--- • ----	/	:	I  :	I  :	I	I \'.,

1' \

••	'f-/	f Cherr.), Hill &au.ft	'

FOlftllM._..

1"9MltQI

Fo""''-"'"

1112",lilUJIIHffolNj

$.etnlnPlri

(IIT.Elup(nlfl;llal)

fliltt'IOl'W.,M.IIMI' TfHW/Re Tlltphe/w!Rllifl

o_

.

II,

f

-	-...._)1

II :I

II  II	1	\	I

-.,v...,

I" • ilO.

i---	•

a	a

ID	41

l,,	/  I  I	I	\	:

-·	\ -'·-.J,1. 'I !	l /	/--

I	I	)	:

1	I

S-l!f\llaJH

r-.,.,_.,.	0

SlrutU,HI

.,_.,,_.....

Zbtc d:

-

0

...	@

- @, ··--i-.	lI	-,;--;,/ /

,"	'" -- -..... / ... Y

, .. ;I,,-

lt•SOOO'

,._,,

lc:.. Ul'J1'11"1

Wal«,,...lfl"

E&trtN:f,...--,

..,..c-tllliaMIP&lf

RlufllolWr,U-.

""-""

--

--

-d-

...

IIJ

--,--

-""""  0w...	@

Bi.dl:0.11.	0	...	e

......

w.iao.11	0	@

--

¥',"»ftOtl!.	0

.c....u..,	@

.......

0

e

Ko11pt,hl••

- -

.	Lflll4G

""'"""

"""'"'".......

....._. ,_

-

P,u

0	CMny	@

--

._	,

0	,..	®

®	®

0

®

-	---

. '

--0--

-	r--

IPL

140

1fRlu..

IIRtrt IHIIDOtlol!lf"IMIIRtalfCL

2.} ktc.lfiaiM1111ft!fflC ,._llldNl lv,-..ad\ltrillllea11CY1.

- .

;.-...-::

,_..

---:-::-:-:-

Surmmc':r Crr(i(ia,(«w,·

!:. = :.,:.-:-=,:-:,.,..=..=..-.::::0-1.u. :=.':'9 ·:;..:::.,.

" -=..:.:.:.:..-_-=Zklf .:.At.:.:=:.!:! :-.:..":e•:.:::i.011sic

, • ..  ,,._111NM'-4.......,.$ui,.nklllfl"topNJD_,.,_,s.ur,,.,._,,,.	4

C$,. ,0.11..,d IOCSll »-1.t dO......rl0.111.(, H_,.d0dl,i,t,r)Cl.200J_.

U.1•ld 1'111"1'tJ' lnlt\11• '-M• 111I UIIMCM• Sill'IWy.

lC

- Contours .are to be al L increments.

- Landscape plan required,

SAMPLE SITE PLAN

21

,1.1..,'.",'dfJil'ltl	PU